Rain or Pumps?

Attributing India's Groundwater Decline District by District

End to end review: from the two raw datasets to the trained model

BITS Pilani PGCP AI and ML capstone, Group 18

Kakara Siva Kumar  |  Rainchwar Parth  |  Vipul Aggarwal

Mentor: Sudharshan Deshmukh

27 September 2026

# The story in one page

If you have time for nothing else before the review, this page is the whole arc.

- We were handed a filtered dataset. BITS supplied 2,759 quality-controlled CGWB wells and the IMD 0.25 degree daily rainfall grid. We built the first working pipeline on exactly that.

- The first merge was rejected. We had snapped each well to the nearest rainfall grid node. The examiner called it out. Measured properly the displacement was 10.2 km on average, not the 55 km claimed, but the step-function error was still worth 10.1 percent of mean monthly rainfall, so we replaced it with bilinear interpolation.

- Then we found the supplied dataset itself was biased. Replaying the source paper’s filter on the raw 32,299-well archive reproduced the 2,759 exactly, and showed one rule removed 25,736 wells for incompleteness rather than for being wrong. The wells it removed are deepening more often. The supplied set understates the decline.

- So we rebuilt from the raw archive. Flag bad readings instead of dropping wells; locate wells by coordinates instead of by district name; cut seasons between readings instead of on calendar quarters; take specific yield from the hydrogeological map instead of a state median.

- The final dataset is a well-season table. One row per well per season, target delta_h_m, the change in water level since the previous reading. Rainfall features only. No past water level anywhere in the inputs, because the residual has to stay interpretable as extraction pressure.

- The model works and the residual path is live. The transformer reaches R² 0.44 on held-out 2018 to 2022, matching LightGBM, and the district residual table is produced end to end.

One sentence to have ready: "We were given a filtered dataset, we proved the filter was biased against exactly the wells that matter, and we rebuilt from the raw archive."

# 1. Stage one: the dataset the problem statement gave us

The BITS problem statement names two sources and says they will be provided as a ready-to-use extract:

| Source | What it is | Size as supplied |
| --- | --- | --- |
| CGWB Quality-Controlled Groundwater Levels 2000-2022 | Seasonal depth-to-water readings at monitoring wells, with specific yield. figshare, CC BY 4.0, doi 10.6084/m9.figshare.29293877.v3 | 2,759 wells, four readings a year, 92 campaigns |
| IMD 0.25 degree Gridded Daily Rainfall | Daily rainfall grid for India, mm per day. imdpune.gov.in, open access | 25 NetCDF files 1998-2022, 129 x 135 = 17,415 points per day |

## What the two files actually look like

This matters because the review will ask how you joined them, and the honest answer starts with the fact that they share no join key at all.

- CGWB is wide. One row per well. Location and construction in the first columns, then 92 columns named Jan-00, May-00, Aug-00, Nov-00 through to Nov-22, each holding a depth in metres below ground level. Point measurements at arbitrary surveyed coordinates.

- IMD is a cube. Dimensions TIME x LATITUDE x LONGITUDE. Latitudes run 6.5 to 38.5 and longitudes 66.5 to 100.0, both in steps of 0.25. Values outside India’s land are coded -999.

- Only 3 of the 2,759 wells sit exactly on a grid node. So rainfall has to be transferred from the grid to the well, and how you do that is a modelling decision, not a plumbing detail.

## The three things about the target nobody tells you

- Depth is measured downwards. A larger number means the water is deeper, which is worse. mbgl is metres below ground level.

- A water level is a stock and rain is a flow. Rain over a period explains the change in level over that period, not the level itself. Every design decision downstream follows from this one sentence.

- Extraction is not in either dataset. There is no pumping column. That is not an oversight in the data, it is the project. The extraction signal is what is left over after rainfall has been accounted for.

Likely question: "Why is this a regression problem and not classification?" Because the target is a continuous change in metres. Classification enters only later, as a validation layer, when the district ranking is cross-checked against CGWB’s over-exploited, critical and semi-critical categories.

# 2. Stage two: the first pipeline, and why it was rejected

## What we built

Weeks 1 and 2 produced a district-season table from the supplied files. Wells were assigned to the nearest rainfall grid node by rounding each coordinate to the nearest multiple of 0.25, rainfall was aggregated to the season, and districts with at least three wells were kept. That gave 22,715 district-season rows over 272 districts.

## The review finding

The examiner rejected the merge on the grounds that one degree is about 110 km, so rounding coordinates could displace a well by tens of kilometres. The arithmetic in the objection was wrong and the substance was right.

| Claim | Measured across the real 2,759-well network |
| --- | --- |
| Displacement up to 55 km | Not attainable. Nearest-node snapping moves a well at most half a cell on each axis, 0.125 degrees |
| Actual mean displacement | 10.2 km |
| Actual median | 10.6 km |
| Actual maximum | 19.4 km |

The substantive point held anyway. Nearest-node assignment is a step function: a well one kilometre inside a cell boundary takes 100 percent of one cell’s rain and nothing from the neighbouring cell one kilometre away. Measured over the full record, that differs from bilinear interpolation by 10.1 percent of mean monthly rainfall. For a project whose entire claim rests on the unexplained part of the level, a 10 percent error in the only observed driver cannot be defended.

## The fix, and the proof it does not change the answer

Rainfall is now sampled at each well’s true coordinates by bilinear interpolation over the four surrounding nodes. Because IMD codes the sea as missing, a naive bilinear would poison every coastal well, so the value and a land mask are interpolated separately and the value is divided by the mask. That is a bilinear average over whichever of the four neighbours are land.

We then re-ran the whole analysis four ways to show the choice does not drive the result:

| Transfer method | Correlation with bilinear | Mean absolute difference | Share of mean rainfall |
| --- | --- | --- | --- |
| Nearest node | 0.9883 | 10.79 mm | 10.1 % |
| 25 km buffer average | 0.9960 | 5.71 mm | 5.4 % |
| 50 km buffer average | 0.9819 | 14.51 mm | 13.6 % |

District rankings from all four methods agree at Spearman above 0.98 in every pairing, and 19 of the top 20 districts are shared between nearest-node and bilinear. So the rainfall numbers are sensitive to the method and the output ranking is not.

The honest bound: IMD’s own grid is built by inverse distance weighting from one to four stations within a 165 km radius (Pai et al., 2014). The 0.25 degree spacing is a presentation resolution, not an information resolution. No transfer method recovers true rainfall at a wellhead, and the feature should be described as a well-centred estimate of a regionally smoothed field.

# 3. Stage three: the rebuild, and why the supplied dataset had to go

This is the part of the story most worth rehearsing, because it is the strongest thing the group did and it is easy to tell badly.

## 3.1 The supplied 2,759 wells are a biased sample

The raw archive behind the published extract holds 32,299 wells. Replaying the source paper’s quality control on it reproduces the published 2,759 exactly, which means we can see precisely which rule removed what:

| Filter step | Wells remaining |
| --- | --- |
| Raw file | 32,299 |
| Has any reading 2000-2022 | 29,537 |
| No negative reading anywhere (drops the whole well) | 28,612 |
| At least two readings in every year 2000-2022 | 2,876 |
| No value repeating more than twice in a row | 2,759 |

The fourth rule alone removes 25,736 wells, and it tests completeness rather than quality. A well with 91 of its 92 readings present is discarded if a single year has only one reading. That would be merely wasteful if the loss were random. It is not: 46 percent of the wells the rebuild adds back are deepening, against 38 percent of the published ones. The supplied dataset systematically understates the decline the project exists to measure.

## 3.2 What changed, item by item

| Problem found in the raw data | What was done instead |
| --- | --- |
| Station codes damaged by scientific notation (1.4113E+14), so they cannot identify a well. 3,710 wells share a coordinate | Key each well by its coordinates as well_uid, with a stable suffix where wells share a site |
| 610 district names mix spellings, renames and boundary vintages (Trichur/Thrissur, Belgaum/Belagavi, Telangana pre- and post-2016) | Place each well in the district outline its coordinates fall inside. 32,205 wells placed directly, 87 snapped to the nearest outline within 5 km, 7 left with no district. CGWB labels kept as cgwb_district for audit |
| 489 wells stand in a different state from the one the file names | Coordinates win. A well’s level correlates with its neighbours at 0.39 and with its labelled district at 0.17, and the coordinates fit better for 69 percent of the conflicts. Disagreement recorded as state_agrees = False |
| Bad readings: negatives, zeros used as missing, water deeper than the well was drilled, repeated runs, outliers | Flag, never delete. Six flags; 59,977 readings invalid, 1,065,392 valid, 94.7 percent of 1,125,369. A plain zero is not invalid on its own because a shallow well can fill to the surface |
| IMD writes 0.0 mm for a whole year in cells with no nearby gauge. Saiha in Mizoram read 0 mm across nine years and produced a false +1,296 mm per decade trend | Flag 8,693 suspect-zero cell-months as missing; averages over cells renormalise over the cells that have data. Saiha corrects to 2,538 mm a year |
| Specific yield was absent from the raw file and had been filled with a state and aquifer median | Take Sy from the hydrogeological map in the archive, five classes from 0.018 aquitard to 0.130 alluvium. Validated: the map returns the published value for 100 percent of the 2,872 wells that fall on a map cell. The old guesses were the wrong class for 8,996 wells, and off by more than 1.5x for three quarters of those |
| Aquifer type blank for 13,312 wells | Dug wells with a recorded type are 99.6 percent unconfined, so blank dug wells are inferred Unconfined; the rest go to Unknown. Source recorded in aquifer_source |

## 3.3 The season clock

Rain is daily and levels are read four times a year, so the two have to be put on one clock. A calendar quarter such as July to September straddles the August reading and mixes rain the level has already answered with rain it has not. Instead a season runs from the day after one reading to the next reading, cut on the 15th:

| Season | From | To | Days | Closed by | Label |
| --- | --- | --- | --- | --- | --- |
| Nov-Jan | Nov 16 | Jan 15 | 61 | January reading | post-monsoon |
| Jan-May | Jan 16 | May 15 | 120 / 121 | May reading | dry |
| May-Aug | May 16 | Aug 15 | 92 | August reading | early monsoon |
| Aug-Nov | Aug 16 | Nov 15 | 92 | November reading | late monsoon |

99 seasons in all, from Jan-May 1998 to Aug-Nov 2022. Season totals are summed from the daily grid, not from a monthly table, because the cut falls mid-month. Checked against the raw NetCDF including a year boundary and a leap year: maximum difference 0.00004 mm.

## 3.4 Putting both in the same unit

Specific yield converts a change in level into a depth of water, which is what makes the rainfall comparison meaningful:

delta_h_m         = depth now  -  depth at previous reading      (positive = water fell)

storage_change_mm = -delta_h_m x Sy x 1000                       (positive = water gained)

recharge_ratio    = storage_change_mm / rain over the same span

| Season | Median level change | Median rain | Share of rain stored |
| --- | --- | --- | --- |
| Nov-Jan | falls 0.65 m | 7 mm | - |
| Jan-May | falls 1.42 m | 39 mm | - |
| May-Aug | RISES 1.94 m | 482 mm | +11.6 % |
| Aug-Nov | falls 0.15 m | 359 mm | about 0 |

The May-Aug figure of 11.6 percent sits inside CGWB’s own published infiltration range of 8 to 15 percent. That is an external validation of the whole chain, and it is worth quoting.

# 4. The mentor suggestion we tested and did not adopt

The mentor proposed an RNN reading 30 days of rain to predict rain on day 31, fed back on itself to forecast forward. It was tested rather than argued about, on 300 random cells over 2018 to 2022.

- The last 30 days predict the next day at R² 0.145, which is worse than the calendar average at 0.17.

- Fed back on its own predictions, skill falls to the calendar average within a week and the forecast spread shrinks to a third of reality.

- More fundamentally, rain at the wells is already observed for the whole study period. It is an input, not a target.

What was kept from the suggestion: distance weighting in the spatial merge, the error cost function, the "rain in, groundwater out" framing, and comparing two architectures. Future scenarios are asked with scenario rain, for example plus 20 percent, rather than with a forecast.

This is a good answer to have ready because it shows the group tested a mentor suggestion empirically and kept the parts that survived, rather than either ignoring it or adopting it uncritically.

# 5. The final dataset we agreed on

One row per well per season that has a usable previous reading. The grain is deliberately the well and not the district: aggregating to the district first throws away within-district variation and shrinks the sample by two orders of magnitude. Districts come back at the residual stage.

## 5.1 The contract

| Element | Definition |
| --- | --- |
| Target | delta_h_m, the depth now minus the depth at the previous reading of the same well, in metres. POSITIVE MEANS THE WATER LEVEL FELL |
| Alternative targets kept | storage_change_mm and anomaly_ref_m, available but not used for the headline model |
| Features | Rainfall over the interval, over 30/60/90/180/365/730 days, monsoon rain, wet days, wettest day, rain per wet day; rain 1, 2 and 3 seasons before; rain over the last 4, 8 and 12 seasons; rainfall normals and anomalies against them; specific yield, drilled depth; categories season, aquifer, well type, rock class |
| Never a feature | Any past water level, the year, latitude, longitude, district, state, well id. The build aborts if one of them is listed as a feature |
| Normals | Computed from training years only, so no validation or test information leaks into a training-time feature |
| Split | Blocked by year. Train 2000-2014, validation 2015-2017, test 2018-2022 |
| Second split | Five folds of whole districts, for testing transfer to districts the model has never seen |
| Missing data | Never filled. A missing reading is bridged from the last valid one up to a year back, with rain summed over the same span and span_seasons recorded as a feature |

Say the sign convention out loud when you present. delta_h_m positive means the level FELL. Get it backwards on a chart and the entire attribution story inverts.

## 5.2 Why no past water level, ever

This is the single most likely technical challenge in the review, so the answer should be immediate.

Groundwater levels are strongly autocorrelated. Feed last season’s depth in as a feature and the model reaches an R² near 0.9 by learning mean reversion. It looks excellent and it is useless, because the residual then measures nothing but the model’s own smoothing error. The project needs the residual to mean "the part of the change that rainfall does not explain". The moment the water level is an input, that sentence stops being true. The guard is enforced in code, not by discipline: there is an assertion in the loader that fails the run if any forbidden column appears in the feature list.

## 5.3 Two tables exist right now, and you should say so

There are two builds of the same contract. They are not in conflict, but a reviewer who sees both will ask.

|  | Full rebuild (Parth) | Direct build (Siva) |
| --- | --- | --- |
| Source | Raw 32,299-well archive | Published 2,759-well CSV |
| Rows with a target | 694,253 | 216,455 |
| Wells | 11,455 in the training view | 2,759 |
| Train / valid / test | 489,707 / 85,250 / 119,296 | 144,558 / 27,774 / 44,123 |
| Features | 25 | 33 |
| Rain sequence | 52 weeks, stored per land cell as a 99 x 52 x 4,964 cube with a stencil | 104 weeks, stored per row |
| Districts | 598 | 272 with three or more wells |
| Needs to run | Raw zip unpacked, districts geojson, geopandas, a manual gap-cleaning notebook | The two supplied files and nothing else |
| Status | Code complete, data not yet materialised locally | Built and trained |

Same target, same forbidden list, same split years, same five district folds, same column names. The direct build exists so the architecture work could start without waiting on the pipeline prerequisites; swapping to the full table is a path change, not a rewrite. Both should be reported, with the full rebuild as the headline dataset and the direct build as the one the current numbers come from.

# 6. What we did with the final dataset

Three model arms read the same table, so any difference between them is the architecture and nothing else.

| Arm | Owner | What it tests |
| --- | --- | --- |
| Logistic regression | Vipul | The classification validation layer against CGWB stress categories |
| BiLSTM, forward and backward | Parth | Whether recurrence over the weekly rain sequence helps |
| Transformer encoder | Siva | Whether attention over the raw weekly sequence beats hand-built rain windows |
| Foundation models | Siva, then joint | Whether a pretrained time-series model transfers to this problem at all |

## 6.1 The transformer, and the reason for every choice

weekly rain (104,)  ->  26 patches of 4 weeks  ->  linear + learned position

->  3 encoder blocks, 4 heads, d=64, pre-norm

->  mean pool                                    (64,)

tabular (29 numeric + 4 categorical embeddings)  ->  MLP             (64,)

concat (128,)  ->  MLP  ->  scalar

176,817 parameters. Small deliberately: 144,558 training rows does not support a large model.

| Choice | Reason |
| --- | --- |
| A transformer at all, when the table already has rain_30d, rain_90d, rain_365d | Those windows are a human guess about which part of the rain history matters. Attention learns the weighting instead of being told it. That is the whole argument |
| Patches of four weeks, not one token per week | 104 tokens costs 104 squared per head for nothing. Rainfall decorrelates in days, so a four-week patch discards no structure, and the attention map then reads as "which month mattered" |
| No causal mask | This is not forecasting. Every one of the 104 weeks is already in the past at prediction time, so the encoder should see all of them at once |
| Pre-norm | Post-norm needs a warmup schedule to train at this depth. Pre-norm simply trains |
| Huber loss rather than MSE | delta_h_m has a long tail of real 10 m swings. Under MSE a handful of them would set every gradient |
| log1p on the rain sequence | Weekly rain is zero-inflated and right-skewed. log1p pulls the 500 mm weeks in without discarding them |
| Mean pool rather than a CLS token | With 26 patches and no pretraining a CLS token has nothing to inherit, and this is one fewer thing to defend |

## 6.2 Foundation models: the finding before the experiment

No pretrained groundwater model exists. Nothing on HuggingFace is trained on groundwater levels. What exists is general time-series foundation models:

| Model | Takes covariates | Verdict |
| --- | --- | --- |
| Chronos / Chronos-Bolt (Amazon) | No | Out. Univariate only, so it cannot see rainfall |
| Lag-Llama | No | Out, same reason |
| TimesFM (Google) | Yes, from 2.x | Candidate |
| Moirai (Salesforce) | Yes, any-variate | Candidate, designed for exogenous inputs |
| Granite-TimeSeries (IBM) | Yes | Candidate, patch-based and close to what we built |

The trap, which should be stated before anyone else raises it. These models forecast a series from its own past. Point one at groundwater depth and it will predict next season’s depth from last season’s, score beautifully, and answer a question nobody asked. Our target is explicitly the part of the level that rain does not explain, so any model given past depth destroys the attribution. A foundation model is admissible here only if depth never enters its input.

The framing that survives is a frozen encoder: run the weekly rain sequence through a frozen TimesFM or Moirai encoder, take the representation, train only a small head on delta_h_m. Depth never enters. It drops into the existing model by swapping the sequence encoder, which makes it a clean comparison against our own 177k-parameter encoder on the same rows and split. A second, strictly separate use is forecasting rainfall to drive the what-if scenario tool, which is useful for the simulator and irrelevant to attribution.

# 7. The outcome

## 7.1 Validation 2015-2017, all models on the same rows

| Model | MAE (m) | RMSE (m) | R² |
| --- | --- | --- | --- |
| Zero, predict no change | 2.163 | 3.227 | 0.000 |
| Season mean | 1.813 | 2.756 | 0.271 |
| Ridge on tabular features | 1.753 | 2.872 | 0.208 |
| LightGBM on tabular features | 1.526 | 2.435 | 0.431 |
| Transformer, sequence plus tabular | 1.531 | 2.461 | 0.418 |
| Transformer, sequence only | 1.641 | 2.618 | 0.342 |

Held-out test 2018-2022, transformer: MAE 1.457 m, RMSE 2.302 m, R² 0.439.

Training R² is 0.513 against 0.418 on validation and 0.439 on test. A gap of 0.07 to 0.10 with no collapse, and test scoring above validation, so the model is not degrading as it moves forward in time. Early stopped at epoch 14 with the best weights from epoch 9.

## 7.2 How to present the LightGBM tie

The transformer and LightGBM are the same model to two decimal places. Claim this, and not a win:

Attention over the raw weekly rainfall series recovers gradient boosting on hand-engineered rain windows, without being told which windows matter.

LightGBM was handed eleven rainfall windows that a human chose. The transformer was handed 104 raw weekly numbers and found its own weighting. Matching a strong baseline from a weaker starting position is a result. Claiming victory on 0.005 m of MAE is not, and it would be taken apart. If pressed on why boosting is not simply better: 216,000 rows of 33 tabular features is boosting’s home ground, and a 177k-parameter transformer on 145,000 training rows is in the regime where deep models have no data advantage. Both statements are true and neither is a defect in the architecture.

## 7.3 The ablation is the more interesting number

Rainfall sequence alone, with no tabular features at all, reaches R² 0.342 against the full model’s 0.418. The rain history recovers 82 percent of the explained variance on its own. That is exactly what the project needs to be true: if the static well properties had dominated, the residual would be mostly well-specific noise and the extraction attribution would be far weaker.

One caution to raise yourself. LightGBM’s feature importances rank well_depth_m first, then three rainfall normals. Normals are per-well constants, so they are partly acting as a well fixed effect, telling the model which well it is looking at without naming it. It is not leakage, because the normals are computed on training years only, but it does mean part of the tabular arm’s contribution is well identity rather than hydrology, which makes the sequence-only result look better still.

## 7.4 Attention

The attention figure shows how much each of the 26 four-week patches is attended to, averaged over 2,000 validation rows per campaign. Read it honestly: uniform attention would be 0.038 and the bars run 0.025 to 0.071, so the structure is real but moderate, not a sharp monsoon spike. The August campaign concentrates around 28 to 30 weeks and 14 to 16 weeks before the reading; the November campaign peaks near a full year back. The first and last patches are elevated in every season, which is a known positional-encoding edge artifact and not hydrology. Saying that yourself is worth more than a cleaner-looking claim.

## 7.5 The residual path is live

Residual is actual minus predicted, so a positive residual means the level fell more than rainfall accounts for, which is the extraction-pressure direction. Averaged by district over wells with at least three monitoring wells, the current top of the list is Junagadh 0.181, Bangalore urban 0.176, Sundargarh 0.160, Ganjam 0.152, Gajapati 0.149, Rayagada 0.147.

Present this as a wiring check, not a finding. No district effects are fitted, no trend is estimated, and several entries rest on three wells. Its purpose is to show the chain runs end to end from raw NetCDF to a district ranking.

# 8. Where everything lives

Everything now sits in one repository, rain-or-pumps. It replaces the scattered Downloads folders and both earlier repositories, and it rebuilds itself with no download: the 607 MB of IMD NetCDF files are packed into a single 22 MB array that is tracked in git.

## 8.1 Three commands

make data      build the 216,455-row training table          about 15 seconds

make train     baselines, transformer, ablation              about 25 minutes

make report    attention figure and district residuals

Also: make cv for the five district folds, make test for the pipeline’s 79 tests, make fetch for the raw inputs, make cube to regenerate the packed rainfall array.

## 8.2 The tree

| Path | What it is |
| --- | --- |
| README.md | The project, the results table, the layout, and where to start reading |
| Makefile | Every task above |
| requirements.txt | One environment for the pipeline and the models |
| data/README.md | What is tracked, what is fetched, and the licence of each source |
| data/derived/rain_cube.npz | TRACKED. The whole IMD record: 9,131 days x 4,964 land cells, tenths of a mm in int16, 22 MB |
| data/cgwb/, data/imd_rainfall/, data/*.zip | Not tracked. Raw inputs, restored by scripts/fetch_data.py |
| data/training/ | Not tracked. tabular.parquet, rain_seq.npz, feature_spec.csv, build_report.txt. Rebuilt by make data |
| data_cleaning/reference/districts.geojson | 724 district outlines, post-2020 boundaries |
| data_cleaning/clean_gaps.ipynb | Specific yield from the hydrogeological map, aquifer inference, reruns the panel and season steps |
| pipeline/bits_ml/config.py | Every path the pipeline reads and writes. Start here when a file is not found |
| pipeline/bits_ml/ingest.py | Step 1. All wells and readings from the raw archive, the six quality flags, location from coordinates |
| pipeline/bits_ml/grid.py | Step 2. Grid cells and months; well to cell and cell to district links |
| pipeline/bits_ml/rain_panel.py | Step 3. Monthly rain per cell; flags the IMD zeros that are really missing |
| pipeline/bits_ml/panel.py | Step 4. Monthly tables at well, cell and district level |
| pipeline/bits_ml/seasons.py | Step 5. The four seasons between readings, level change, storage change |
| pipeline/bits_ml/rainfall.py | NetCDF reader and the masked bilinear interpolation |
| pipeline/bits_ml/districts.py | District outlines and area-weighted cell to district mapping |
| pipeline/bits_ml/training_data.py | Step 8. The 694,253-row table, 25 features, splits, weekly rain sequences |
| pipeline/tests/ | 79 tests across ingest, grid, rainfall, panel, seasons, districts and training_data |
| models/build_training_data.py | The direct build: CGWB extract plus the packed cube to data/training. Falls back to the NetCDFs when the cube is absent |
| models/transformer.py | SeqEncoder, TabEncoder, RainTransformer, SeqOnlyTransformer, and attention_maps |
| models/train_transformer.py | Baselines, training loop, ablation, prints the comparison table |
| models/attention_report.py | Loads the trained model, writes the attention figure and the residual table |
| models/artifacts/transformer.pt | Trained weights, best epoch |
| models/artifacts/results.json | Every metric in section 7, plus the per-epoch history |
| models/START_HERE.md | The contract and the commands |
| models/RESULTS.md | The numbers in section 7, written up |
| notebooks/01_eda_feature_engineering.ipynb | Week 1 exploration and feature engineering, executed |
| notebooks/02_clean_gaps.ipynb | The gap-closing notebook, step 6 of the pipeline |
| docs/00_problem_statement.docx | The brief as issued |
| docs/01_execution_plan.docx | The eight week plan |
| docs/02_week1_code_walkthrough.docx | Line by line walkthrough of the week 1 notebook |
| docs/03_glossary.docx | mbgl, Sy, and the rest |
| docs/04_spatial_merge_methodology.docx | The bilinear justification and the sensitivity test, with citations |
| docs/05_end_to_end_review.docx | This document |
| docs/data_cleaning/DATA_CLEANING_SUMMARY.txt | Every cleaning decision and its evidence. The most useful single file in the repository |
| docs/data_cleaning/DATASETS.md, MODELLING_GAPS.md | Every table and its role; the mentor suggestion tested empirically |
| decks/ | The review presentations |
| reports/figures/attention_by_season.png | The attention figure |
| reports/residual_by_district.csv | District residual table |
| reports/spatial_sensitivity_rankings.csv | District rankings under all four spatial transfer methods |
| scripts/fetch_data.py | Restores the raw CGWB and IMD inputs |
| scripts/build_rain_cube.py | Packs the NetCDFs into data/derived/rain_cube.npz |

## 8.3 Full rebuild from the raw archive

Only needed for the 694,253-row table. The direct build does not require any of this.

python3 scripts/fetch_data.py --all

cd pipeline

python -m bits_ml.ingest -> grid -> rain_panel -> panel -> seasons

then notebooks/02_clean_gaps.ipynb, then python -m bits_ml.training_data

Why the rainfall record is packed rather than tracked as NetCDF: the IMD files store a value for every point of a 129 x 135 grid and 12,451 of those 17,415 points are sea. Only 4,964 are land. Land columns only, tenths of a millimetre in int16, and 72 percent of the entries exactly zero, compresses 607 MB to 22 MB. Measured against reading the NetCDFs directly over all 2,759 wells and all 9,131 days: maximum difference 0.05 mm, mean difference 0.005 mm. Both code paths are in the builder, so the check can be repeated.

# 9. Questions to expect, and the short answers

| Question | Answer |
| --- | --- |
| Why is extraction not a feature? | It is not in either dataset, and it is the thing being estimated. Making it an input would be circular. The residual is the estimate. |
| How did you join two datasets with no common key? | By coordinates. Rainfall is sampled at each well’s true latitude and longitude by bilinear interpolation over the four surrounding grid nodes, with the land mask interpolated separately and used to renormalise so coastal wells are not poisoned by sea cells. |
| Why not snap to the nearest node, it is simpler? | It is a step function, and it differs from bilinear by 10.1 percent of mean monthly rainfall. We measured it. The district ranking survives either way at Spearman above 0.98, but the input error does not survive scrutiny. |
| How accurate can the rainfall at a well really be? | Bounded by the source. IMD interpolates each grid value from one to four stations within 165 km, so the field is smooth by construction. The feature is a well-centred estimate of a regionally smoothed field, and we say so. |
| Why did you not use the dataset you were given? | We did, first. Then we showed its filter removes 25,736 wells for incompleteness rather than for error, and that the removed wells are deepening more often, so the supplied set understates the decline. We kept it as a control group. |
| Is there leakage? | The forbidden list is enforced by an assertion that fails the build. No past water level, no year, no coordinates, no district, no state, no well id. Normals are computed on training years only. The split is blocked by year. |
| Why regression and not classification? | The target is a continuous change in metres. Classification enters as a validation layer, checking the district ranking against CGWB’s stress categories. |
| Why is R² only 0.44? | Because rainfall alone should not explain a groundwater change. Season rain correlates with level change at only -0.08 to -0.16. The unexplained part is the signal, not the failure. Pumping, cropping pattern and canal supply are not in the data by design. |
| Your transformer did not beat LightGBM. | Correct, it matched it. LightGBM was given eleven rainfall windows chosen by a human; the transformer was given 104 raw weekly numbers and found its own. Matching from a weaker starting position is the result we are claiming. |
| What does a positive residual mean? | The level fell more than rainfall accounts for. That is the extraction-pressure direction. |
| Why 104 weeks? | Two years, so the encoder can see the previous monsoon as well as the current one. The 52-week ablation is one command and is on the list to run. |
| What is still missing? | The district-fold cross-validation run, transfer to unseen districts, the frozen-encoder foundation-model comparison, and materialising the full 694,253-row table locally. |

# 10. Open gaps, stated before they are found

| Gap | Size | How it is handled |
| --- | --- | --- |
| Rain explains little on its own | Season rain vs level change correlates -0.08 to -0.16 | Expected. The rest is the signal we are after |
| Reading day unknown | Every reading | Placed on the 15th. Season boundaries uncertain by about two weeks. Stated |
| Thin survey rounds | May 2020 and May 2021 at 1 percent of wells read | Left missing, bridged from the last valid reading |
| Changes spanning several seasons | 17 percent of training rows | span_seasons is an input feature; rain is summed to match |
| Extreme changes | 0.24 percent beyond plus or minus 20 m | Huber loss rather than MSE |
| Wells drop out over time | 10,619 in 2005 down to 8,053 in 2022 | Also score on a stable subset |
| Specific yield is a class, not a measurement | All wells | Stated. delta_h_m in metres is unaffected |
| Wells are not a random sample | About 80 percent shallow dug wells | Stated. Spatial precision matters most for exactly this kind of well, which is why the merge was tested |
| Aquifer type unknown | 468 modelling wells | Kept as Unknown rather than imputed |

## Next steps

- Run the five district folds and report transfer to unseen districts. This is the harder and more relevant generalisation claim for a district-level product.

- Run the 52-week, single-patch and single-layer ablations so every architectural choice has a number behind it.

- Materialise the full 694,253-row table and re-run the model on it. Path change only.

- Frozen-encoder comparison against TimesFM or Moirai, same head, same rows, same split.

- Fit district effects properly and estimate residual trends, which turns the wiring check in section 7.5 into the actual attribution output.