# Glossary of Terms and Abbreviations

Rain or Pumps? Attributing India’s Groundwater Decline District by District

Sivakumar Kakara, BITS Pilani PGCP AI/ML Capstone

Table of Contents

# 1. The five terms you must not fumble

| Term | Full form | What it means here |
| --- | --- | --- |
| gwl_mbgl | Groundwater level, metres below ground level | Our target variable. Distance downward from the ground surface to the water table. 4.6 means water sits 4.6 m under your feet. Bigger = deeper = worse. A decline shows as an increase. |
| CGWB | Central Ground Water Board | The Government of India agency (under the Jal Shakti ministry) that monitors India’s groundwater. Source of our well data and of the official stress categories we will validate against. |
| IMD | India Meteorological Department | The national weather agency, Pune. Source of our gridded rainfall data. |
| JJAS | June July August September | The southwest monsoon season. About 79% of annual rainfall at our well sites falls in these four months, so it is the dominant recharge period. |
| Sy (Reference_Sy) | Specific yield | The fraction of water an unconfined aquifer releases per unit drop in the water table. Roughly its storage capacity. Two aquifers with the same recharge show different level responses if Sy differs. |

# 2. Hydrology and domain terms

| Term | Full form / meaning |
| --- | --- |
| Water table | The upper surface of the saturated zone underground. What our wells measure the depth to. |
| Aquifer | An underground layer of rock or sediment that holds and transmits water. |
| Unconfined aquifer | An aquifer open to the surface, so rainfall reaches it directly. 93% of our wells sit in these, which is why levels respond to rainfall within months. |
| Confined aquifer | An aquifer sealed under an impermeable layer. Responds slowly and indirectly, so it is harder to link to recent rainfall. |
| Semi-confined | Partially sealed, intermediate behaviour. |
| Recharge | Water entering the aquifer. Dominated by monsoon rainfall; also canal seepage and irrigation return flow (neither observed in our data). |
| Extraction / abstraction / draft | Water pumped out. Three words for the same thing; CGWB reports use “draft”. Irrigation is roughly 85 to 90% of India’s groundwater draft. |
| Pre-monsoon | Before the rains, measured in May. The annual maximum depth (driest state). |
| Post-monsoon | After the rains, measured in November. The annual minimum depth (fullest state). This is the standard season for assessing groundwater status. |
| Dug well | A wide, hand-dug shallow well. 93.5% of our monitoring network. Responds quickly to rainfall. |
| Bore well / tube well | Narrow drilled wells, usually deeper, often tapping confined layers. |
| Hard rock terrain | Crystalline basement rock (peninsular India). Low storage, so levels swing sharply. |
| Alluvium | Loose river-deposited sediment (Gangetic plain). High storage, shallower and more stable tables. |
| Baseflow | Groundwater discharging naturally into rivers. A natural outflow, not extraction. |
| Evapotranspiration | Water lost to the atmosphere from soil evaporation plus plant transpiration. |
| Command area | The land served by a canal system. Canal water can mask or offset groundwater decline, which is a confounder for us. |
| Boro paddy | The dry-season (winter to spring) rice crop of eastern India, heavily groundwater-irrigated. Explains rising extraction in West Bengal and Bihar districts. |
| Green Revolution belt | Punjab and Haryana’s intensive paddy-wheat region. The classic over-extraction story; our top decliners sit here. |
| Assessment unit | The block, taluk or mandal that CGWB scores when assigning stress categories. |
| Safe / Semi-critical / Critical / Over-exploited | CGWB’s four stress categories, based on the ratio of annual extraction to annual recharge (broadly: under 70%, 70-90%, 90-100%, over 100%). Roughly 67 / 15 / 4 / 14 percent of units nationally. |
| ENSO | El Niño Southern Oscillation. A Pacific Ocean climate cycle that strongly influences the Indian monsoon. El Niño years tend to be drought years. |
| IOD | Indian Ocean Dipole. A similar sea-surface temperature oscillation in the Indian Ocean, also a monsoon driver. |
| Niño 3.4 | A specific Pacific region whose sea-surface temperature anomaly is the standard numeric ENSO index. Would be the natural exogenous input if we extend the rainfall model. |

# 3. Data formats and sources

| Term | Full form / meaning |
| --- | --- |
| CSV | Comma-separated values. Plain-text table. Our CGWB file and final data set. |
| NetCDF (.nc) | Network Common Data Form. A binary format for multi-dimensional scientific arrays. Each IMD file holds RAINFALL indexed by [time, latitude, longitude]. |
| Parquet | A compressed, column-oriented binary table format. Loads far faster than CSV and preserves data types. Used for our intermediate tables. |
| .ipynb | IPython notebook. The Jupyter notebook file format (it is really just JSON). |
| QC | Quality controlled. The figshare dataset is CGWB’s raw network already filtered for reliability by the dataset authors. |
| DOI | Digital Object Identifier. A permanent citation link for a dataset or paper. |
| CC BY 4.0 | Creative Commons Attribution 4.0. A licence permitting reuse provided the source is credited. |
| figshare | An open repository where researchers publish datasets. Where the CGWB extract lives. |
| 0.25 degree grid | The IMD rainfall resolution. One cell is about 27 km on a side at Indian latitudes. |

# 4. Column names in our data

| Column | Meaning |
| --- | --- |
| well_id | Our minted integer key for each well (row index). Replaces the corrupted Station Code. |
| cell_id | String key for a rainfall grid cell, formed as “latitude_longitude”, e.g. “14.25_78.0”. |
| grid_lat, grid_lon | The well’s coordinates rounded to the nearest grid node. |
| season | The measurement campaign: Jan, May, Aug or Nov. |
| season_col | The original wide column name, e.g. “Aug-07”. |
| month_num, cal_month | Calendar month as a number (1 to 12). |
| date | Datetime stamp of the reading, set to the first of the month. The temporal join key. |
| rain_mm | Total rainfall in millimetres for one cell in one month. |
| wet_days | Count of days in that month with rainfall of at least 2.5 mm (IMD’s rainy-day threshold). |
| rain_3m … rain_36m | Trailing sums of monthly rainfall over the last 3, 6, 12, 24 or 36 months. |
| wetdays_12m | Trailing 12-month count of rainy days. |
| rain_12m_z | Z-score of the 12-month total against that cell’s own long-run average for that calendar month. Separates “dry place” from “dry year”. |
| jjas_mm | Total June to September rainfall for a cell in a given year. |
| jjas_z | The same, standardised against that cell’s own monsoon history. |
| last_monsoon_mm, last_monsoon_z | The most recently completed monsoon’s total and anomaly, attached to each reading. |
| monsoon_year | Which year’s monsoon counts as “last completed” for a given reading. Jan, May and Aug readings use the previous year; only Nov uses the current year. |
| gwl_lag1y | The same well’s same-season reading one year earlier. Aquifer memory. |
| gwl_mean, gwl_median, gwl_std | District-level average, median and spread of depth across that district’s wells. |
| n_wells | How many wells contributed to that district row. A quality indicator. |
| c_mean, c_std | The climatological mean and standard deviation used to build the z-scores. |

# 5. Statistics and machine learning

| Term | Full form / meaning |
| --- | --- |
| EDA | Exploratory data analysis. Understanding data through summaries and plots before modelling. |
| FE | Feature engineering. Creating new input columns from existing data. |
| Target / dependent variable | The quantity being predicted. Ours is gwl_mbgl (well level) or gwl_mean (district level). |
| Feature / predictor / independent variable | An input used to predict the target. |
| Regression | Predicting a continuous number. Our core problem. |
| Classification | Predicting a category. Our validation layer (CGWB stress categories). |
| Residual | Observed value minus predicted value. In this project, the residual from the rainfall-only model is our proxy for extraction pressure. |
| Counterfactual | What would have happened under different conditions. Here: what the level would be given rainfall alone. |
| Baseline | A deliberately simple reference model. A new model is only worth having if it beats the baseline. |
| Climatology | The long-run average for a given location and calendar month. Used as our rainfall baseline. |
| Persistence | The naive forecast “next value equals last value”. Another baseline. |
| Leakage | Accidentally giving a model information it would not have at prediction time (typically future data). Our rule: every feature must be computable on or before the measurement date. |
| Imputation | Filling in missing values. We deliberately do not impute the target. |
| NaN | Not a Number. Pandas’ marker for a missing value. |
| Lag feature | A past value of a variable used as an input. |
| Rolling window | A calculation over the last N periods, recomputed at every step. |
| min_periods | Pandas argument requiring N observations before a rolling result is produced; otherwise NaN. Prevents misleading partial sums. |
| Z-score | (value minus mean) divided by standard deviation. Expresses “how unusual” in units of standard deviations. |
| Skewness | Asymmetry of a distribution. Our target is right-skewed (a long tail of very deep wells). |
| log1p | log(1 + x). A log transform that handles zeros safely; used to make the skewed target near-normal. |
| Pearson correlation | Measures linear association. Sensitive to outliers and skew. |
| Spearman correlation | Correlates ranks, so it measures monotonic association. Robust to skew and outliers. Our choice. |
| Autocorrelation (lag-1) | Correlation of a series with itself one step earlier. Near 0.1 for annual monsoon totals, meaning almost no year-to-year memory. |
| Simpson’s paradox | When a relationship visible within groups reverses or weakens when groups are pooled. Our pooled versus within-district correlation is an instance. |
| Mutual information (MI) | A model-free measure of how much knowing a feature reduces uncertainty about the target. Captures non-linear dependence that correlation misses. Zero only under full independence. |
| Random forest (RF) | An ensemble of decision trees, each trained on a random subset of rows and features; predictions are averaged. Used here only as a quick relevance screen. |
| Feature importance | How much a feature contributed to a tree model’s splits. |
| One-hot encoding | Turning a categorical column into several 0/1 columns, one per category. Applied to season. |
| random_state / seed | A fixed number that makes “random” operations reproducible. Ours is 42. |
| Train / test split | Holding data back to test on. For time series it must be chronological, never random. |
| CV | Cross-validation. Repeatedly training and testing on different data slices to get a stable performance estimate. |
| GroupKFold | Cross-validation that keeps all rows of a group (for us, a district) entirely in train or entirely in test, so the model is tested on unseen districts. |
| Blocked split | Time-based CV, e.g. train on 2000 to 2017 and test on 2018 to 2022. |
| MAE | Mean absolute error. Average of absolute errors, in the target’s own units. Robust to outliers. |
| RMSE | Root mean squared error. Squares errors first, so large errors dominate. |
| R squared | Coefficient of determination. The fraction of variance the model explains; 1 is perfect, 0 is no better than predicting the mean. |
| Overfitting | Learning noise in the training data so performance collapses on new data. |
| Regularisation | Penalising large model coefficients to reduce overfitting. |
| Ridge | Linear regression with an L2 (squared-coefficient) penalty. |
| Lasso | Linear regression with an L1 (absolute-coefficient) penalty; can drive coefficients to exactly zero. |
| ElasticNet | A blend of Ridge and Lasso penalties. |
| Gradient boosting | Building trees sequentially, each correcting the previous ensemble’s errors. |
| XGBoost | eXtreme Gradient Boosting. A widely used, fast gradient boosting library. |
| LightGBM | Light Gradient Boosting Machine. Microsoft’s gradient boosting library, efficient on large data. |
| ANN | Artificial neural network. |
| MLP | Multi-layer perceptron. The basic feed-forward neural network; our planned deep learning benchmark. |
| LSTM | Long Short-Term Memory. A recurrent neural network designed for sequences. |
| PCA | Principal component analysis. Reduces many correlated features to a few uncorrelated components. |
| K-Means | A clustering algorithm that partitions data into k groups around centroids. |
| Class imbalance | When some categories are far rarer than others. CGWB categories are roughly 15:1 imbalanced. |
| SMOTE | Synthetic Minority Over-sampling TEchnique. Creates synthetic examples of rare classes rather than duplicating them. |
| Class weights | Telling the model to penalise errors on rare classes more heavily. Usually the first thing to try before resampling. |
| Precision | Of the cases predicted positive, how many really are. |
| Recall | Of the truly positive cases, how many were caught. |
| F1 | The harmonic mean of precision and recall. |
| PR-AUC | Precision-Recall Area Under the Curve. The right headline metric under heavy imbalance, where plain accuracy is misleading. |
| ROC-AUC | Receiver Operating Characteristic AUC. Can look flatteringly high when classes are imbalanced. |

# 6. Time series terms

| Term | Full form / meaning |
| --- | --- |
| AR | Autoregressive. The value depends on its own previous values. |
| MA | Moving average. The value depends on previous forecast errors. |
| I | Integrated, meaning differencing: modelling changes rather than levels, to remove trend. |
| ARIMA | AutoRegressive Integrated Moving Average. Combines the three ideas above. |
| SARIMA | Seasonal ARIMA. Adds a second set of AR, I and MA terms at the seasonal lag. |
| SARIMAX | SARIMA with eXogenous regressors: external inputs such as an ENSO index. Also the name of the statsmodels class we call (used here without exogenous inputs). |
| Order (p, d, q) | The non-seasonal AR order, differencing order and MA order. Ours is (1, 0, 1). |
| Seasonal order (P, D, Q, s) | The same three at seasonal lag s. Ours is (1, 1, 1, 12): s = 12 because monthly data repeats yearly. |
| Stationarity | A series whose statistical properties do not change over time. ARIMA models assume it, which is why differencing exists. |
| enforce_stationarity=False | Relaxes the constraint that parameters stay in the stationary region during fitting. Helps the optimiser converge across many heterogeneous district series. |
| Seasonality | A repeating cycle of fixed period. In Indian rainfall, the annual monsoon cycle, which dominates everything else. |
| Backtest | Testing a forecasting method on historical data as if forecasting forward, without using future information. |

# 7. Engineering and delivery

| Term | Full form / meaning |
| --- | --- |
| API | Application Programming Interface. A defined way for one program to call another. |
| REST | REpresentational State Transfer. The common HTTP-based API style (GET, POST, and so on). |
| Endpoint | A single callable URL in an API, for example /predict. |
| FastAPI | A modern Python web framework for building REST APIs. Our planned serving layer. |
| LLM | Large language model. |
| RAG | Retrieval-Augmented Generation. Retrieving relevant documents and giving them to an LLM so its answer is grounded in real sources rather than invented. |
| Embedding | A numeric vector representing text, so similar meanings sit close together. The basis of retrieval. |
| Chunking | Splitting long documents into passages small enough to embed and retrieve. |
| LangChain | A Python framework for building LLM applications. |
| LangGraph | A LangChain companion for building agents as explicit graphs of steps, which makes control flow inspectable. |
| Agent | An LLM that decides which tools (here, our API endpoints) to call to answer a question. |
| Tool calling | The mechanism by which an LLM invokes a function and uses its returned data. |
| pandas | Python library for tabular data. |
| numpy | Python library for numerical arrays. |
| matplotlib / seaborn | Plotting libraries; seaborn is a statistical layer on top of matplotlib. |
| scikit-learn (sklearn) | The standard general-purpose Python machine learning library. |
| xarray | Library for labelled multi-dimensional arrays; used to read NetCDF. |
| statsmodels | Python library for statistical models, including SARIMAX. |
| Jupyter kernel | The Python process that executes notebook cells. “Restart and Run All” clears it and re-runs everything from scratch. |

# 8. Course and programme

| Term | Full form |
| --- | --- |
| BITS | Birla Institute of Technology and Science, Pilani |
| PGCP | Post Graduate Certificate Programme |
| AIML | Artificial Intelligence and Machine Learning |
| Capstone | The final integrative project applying all taught topics |
| Mid-sem / End-sem | The week 4 and week 8 evaluation checkpoints |